Typical MGA Twin Cam Weber settings, as reported by owners from the Yahoo Group Site.      -  V0.44 Draft – 27/1/2017 M Hester

Please Note: These are just settings provided by Group Members and come with no warrantee

| Yahoo Group Message ID | MGA Twin Cam cc's and Cams | Weber Type | Comp. Ratio | Est. BHP | Main Venturi | Aux. Venturi | Main Jet | Emulsion Tube | Air Corrector Jet | Idle Jet | Acc. Pump Jet | Pump Ex. Valve | Float Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Factory Settings |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Factory | 1588cc Std Cams | DCOE 40 |  |  | 32 | 3.5 | 125 | F9 | 170 |  | 65 |  |  |
| Factory 2 | 1588cc Std Cams | DCOE 42 |  |  | 32 | 3.5 | 130-135 | F16 | 175 | 50F9 | 45 | 35 | 8.5mm |
| Factory INSKY | 1588cc INSKY TCM Cams | DCOE 42 |  |  | 34 | 3.5 | 155 | F16 | 200 | 55F2 | 55 | 45 | 6.5mm |
| Factory Bob Olthoff | 1762cc, 42mm Ports | DCOE 42 |  |  | 36 | 3.5 | 165 | ? | 240 | 55?? | 65 |  | ? |
| 1598 cc Engines |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21165 | 1588cc Std Cams | DCOE 40 ?? | 9.9 |  | 32 | ?? | 125 | F9 | 175 | 50F8 | 65 | ??? | ??? |
| 21166 http://www.webcon.co.uk/ | 1588 cc Std Cams | ??? |  |  | 30 | 4.5 | 115 | F11 | 200 | 45F9 | 50 | 50 | 12mm |
| 21167 | 1588 cc Std Cams | DCOE 40 | 9.0 |  | 32 | 3.5 | 125 | F9 | 170 | 55F6 | 65 | 50 | ?? |
| 2157 | 1588 cc Std Cams | ??? | 9.1 |  | 32 | 4.5 | 125 | F9 | 170 | 55F6 | 65 | 50 | ?? |
| Dan Casey in 2010 | 1588cc 9.4:1 Arias Pistons, head skimmed 9:9:1 | 40 DCOE 35/34 | 9.9 | TBA | 32 | 4.5 | 125 | F11 | 170 | 50F8 | 40 | 50 | Start Jet F1 120 |
| 30876 Scott Johnston Jim Alcorn Engine | 1588 cc +60 “ Std Cams, ported Head | 40 DCOE 151 | 8.8 | 116 | 30 | 4.5 | 115 | F11 | 200 | 45F9 | 50 | 40 |  |
| None Standard cc Engines |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19878 & 22308 Engine 1 | 1798cc 3brg | DCOE 40 151 |  | 143 BHP | 33 | 3.5 | 135 | F11 | 200 | 55F9 | 40 |  |  |
| 19878 & 22306 Engine 2 | 1670 cc | DCOE 40 151 | 10 | 134 BHP | 33 | 3.5 | 135 | F11 | 180/200 | 45F9 | 40 |  |  |
| Dirk’s Road Engine | 1640 cc Vacuum Distributor 37mm exhausts & 41.5mm inlet + L14 Cams | Dellorto 40 | 10.5 | 136 BHP | 35 | 4.5 | 148 | 7772.10 |  |  | 40 |  |  |
| Dirk’s Race Engine | 82.5mm Pistons 37mm exhausts & 42mm inlet + Cams | DCOE 45 | 11.5 | 177 BHP | 38 | 4.5 | 155 | F16 | 165 | 55F8 | 35 Long rod |  |  |
| Phil’s Road Engine Note: Flat Spot at around 2500 revs. Timing 15° @ Idle 36° Max Adv | BMW Mahle 83.0mm Pistons Standard Valves & Cams | DCOE 40 | 8.3 or 9.0 ?? | 117 BHP | 34 | 4.5 | 125 | F2 | 180 | 55F9 | 40 |  | starter jet F5 100 |
| 23943 Bill Spohn oversize valves, Good power 4500-7800 | Ross Racing 83.5mm pistons, Cosworth L1 cams | DCOE 45 | 12.0 | 180 BHP | 35 | 4.5 | 150 | F16 | 150 | 50F6 | ? | ? | ? |
| Mark Hester | 1924cc 42mm Inlet Valves – LeMans Inlet Cam | DCOE 40 28 | 10.0:1 | ~150 BHP | 34 | Unknown maybe 4.5 | 140 | F16 | 165 | 50F8 | 40 | 70 |  |
| Mark Hester | 1924cc 42mm Inlet Valves – LeMans Inlet Cam | DCOE 40 28 | 10.0:1 | 88 of 160 BHP | 34 | Unknown maybe 4.5 | 170 | F2 | 180 | 60F6 | 60 | 70 |  |

|  | 1. Intake and discharge valve for accelerator circuit 2. Idle jet assembly 3. Main jet stack 4. Starter jet assembly 5. Accelerator pump jet 6. Idle jet 7. Air Correction jet 8. Emulsion tube 9. Main jet 10. Start jet (Cold start circuit) |
| --- | --- |