MGA Twin Cam Production Changes v0.60

Last Updated May 2012

The following amendments, recommendations and other details relate to the construction and maintenance of the MGA Twin Cam. It is by no means a complete document. If an item has been omitted, it may not have seemed vitally important. But there also exists the possibility that full and precise details have not been found. It is to be hoped that this will be remedied in future editions should such information become available.

This Document has grown out of these three documents linked here:

These maybe copyrighted, so these links may need to be removed.

and   and

http://www.angelfire.com/rant/mgatwincam/servicememos/IndexMG.pdf

| Date | Topic | Description | Change Effective |  | Service Memo # |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Car # | Engine # |  |
| April 1958 | Commencement of production | Assembly of the first production Twin Cam YD1/501 was commenced on 22 April 1958. Assembly of the first production Twin Cam engine 16G/U/101 was no doubt undertaken shortly before that date. | 501 | 101 | - |
| June 1958 | Radiator | Overflow pipe modified. Commenced Engine 131 to chassis 651 except Chassis (575, 613, 623, 633, 648) | - | 131 | SPL 3rd Ed – C.1 |
|  | Valve Timing | Change of camshafts and valve timing (see text or see Clausager article. No corresponding text appears in Clausager nor notation in Parts Manual. This will take some investigation. | - | 141 | - |
|  | Thermostat | Thermostat with lower opening temperature. Unsupported by Parts Manual. Change may be other than just temperature. | - | 194 | SPL 3rd Ed – A.16 |
|  | Engine Mount Spacer | A packing plate was added to the engine mountings in order to increase the clearance between the crankshaft starting dog and the steering rack housing. | 528 | - | WM pA18 |
|  | Air Cleaner | Air cleaner with positive filter element location | 528 | - | - |
| 21 July 1958 | Clutch Driven Plate Assembly | As part of a standardization exercise, all BMC clutch plates between 7 ¼ inch and 11 inch diameter were changed from 6 to 4 spring construction. Plates remained interchangeable. | - | - | - |
| September 1958 | Inner Wheel Arches | The wheel arches were fitted with detachable panels in order to facilitate access to the engine compartment. Introduced on coupe from 594. | 592 | - | SPL 3rd Ed – R.1, R.4 |
| October 1958 | Connecting Rod Assemblies | Complete sets of connecting rod assemblies for replacement should all be of similar weight. It is essential that rod assemblies not differ by more than 3 drams (5.316 grammes). This SM was later superseded by SM MG/236 5 November 1958 and then again by SM MG/252 24 April 1959. To facilitate identification, assemblies not to standard weight are stamped to indicate drams above or below standard. Successive SMs introduced additional weight ranges. | 652 | - | MG/231 |
|  | Oil Pump | Layshaft oil pump gear and oil pump spindle with more teeth. | - | 313 | SPL 3rd Ed – A.6 |
|  | Knock-Ons | Octagonal spinners on cars for Germany. | 708 | - | SPL 3rd Ed – P1 |
|  | Speedo Cable | Longer speedometer cable on LHD cars. | 713 | - | SPL 3rd Ed – O.1 |
|  | Temp Gauge | It was, however decided that the water Temperature gauge on Left-Hand-Drive cars not for North America, should be altered from Fahrenheit to Centigrade. (cp. 1500 car number 57574) | 713 | - | SPL 3rd Ed – O.2 |
| November 1958 | Header Tank Cap / Water Loss | Due to the vibration of the valve in the header tank pressure cap, the coolant system suffered significant water loss. This was remedied by the incorporation of a remote 7 lb valve c/w bracket mounting, a new filler cap, (without valve) for the header tank, a rubber connecting hose to join the new valve to the header, and an overflow pipe. In order to fit this system it was necessary to revolve the neck of the header tank through 180 degrees. All cars after chassis 652 were subject to the change, as were, 575, 613, 623, 633 and 648. Kits were made available for earlier versions. Non pressurised cap fitted to header tank. Relief Valve AHH5903 and cage assembly were added. | 652 | 131 | MG/234 & SPL 3rd Ed – C.1, C.2 |
|  | Headlights | Asymmetric headlights introduced on cars from Europe, except for France and Sweden (cp. 1500 car number 58918) | 824 | - | SPL 3rd Ed – N.5 |
|  | Brake Callipers Modified | Front and Rear disc brake calliper units were modified with several parts, including bridge pipes. | 836 | - | SPL 3rd Ed – H.3, K.2 |
|  | Brake Rubber Seals | At this time a vast number of master cylinder and wheel cylinder seals were returned to the Factory under warranty in a swollen and distorted condition. It was stated that these conditions were undoubtedly caused by grease or oil contamination (mineral), or the use of inferior brake fluids. It was then stressed that only the correct/recommended fluids should be used, and that under no circumstances should mineral based oil or grease be permitted to contact any of the hydraulic cylinder components | - | - | MG/235 |
|  | Piston Oil Ring | A twin segmented scraper ring was fitted to reduce oil consumption. An improvement on the earlier design. | - | 446 | SPL 3rd Ed – A.2, A.3 |
|  | Dynamo Bracket | Dynamo adjusting link changed. | - | 448 | SPL 3rd Ed – A.17 |
| December 1958 | Dynamo Pulley | To reduce the battery charging rate, a larger diameter dynamo pulley and longer fan belt were introduced. Old Pulley has the part number AEH504 cast unto it. New pulley is AEH605. Also Fan belt size change (SPL 3rd Ed – A.18) Blake Urban explains more here | - | 272 | MG/237 MG/237A & SPL 3rd Ed – A.17 |
|  | Tonneau Fasterners | Tonneau cover fasteners deleted from door tops and tonneau cover zip lengthened.(cp. 1500 car number 60637) | 994 | - | SPL 3rd Ed – R.49 |
|  | Master Cylinder Box | Brake master cylinder box and strengthening plate modified. Two tapered shims AHH 5874 were deleted. SPL Page M.2 http://www.mgaguru.com/mgtech/twincam/tc206.htm | 997 | - | SPL 3rd Ed – M.1, M.2 |
|  | Ignition Cables Routing | According to the Service Parts List (SPL), ENGINES prior to and including number 556 should have the early style routing with a P clip holding the wires to the middle cam cover bolt. The Ignition cable lengthened were increased and re-routed around front of cylinder head. | - | 556 | SPL 3rd Ed – A.10, A.12, B.2 |
|  | Water Pipe | Water inlet pipe straightened. (Pump to Elbow) | - | 574 | SPL 3rd Ed – A.16 |
|  | Engine Breather | Cylinder block rear side cover breather pipe modified to prevent oil leaking on to exhaust system. I think this is wrong and is the same as 657 | - | 574 657?? | - |
|  | Water Pipe | Water inlet elbow straightened. | - | 581 | SPL 3rd Ed – A.16 |
|  | Top Piston Ring | Modified top ring on piston to overcome piston noise. | - | 606 | SPL 3rd Ed – A.2 |
|  | Conrods | Connecting rod assembly’s. Refer to 29th April 59. | - | - | MG/236 |
| February 1959 | Fuel Octane | Due to a number of problems, it was deemed necessary to advise owners to use 95 as opposed to 90 octane fuels. It was stressed that for competitions and other arduous work, 100 octane should be used. | - | - | MG/238 |
|  | Oil Pump Drive Spindle & Half Speed Shaft | In order to reduce the wear on the gear teeth, the number of teeth on the oil pump drive spindle was increased from 10 to 11. The angle of the teeth on the half speed shaft gear was modified to suit. The change was effected from engine No. 315, but included 313. Interchangeable Together. | - | 315 | MG/240 |
|  | Front Suspension | Due to excessive spring noise the spigot which locates the upper end of the front suspension coil spring was increased in dia. from 2.66 ~ 2.67" to 2.74 ~ 2.75". (cp. 1500 car number 58996). | 842 | - | MG/240 |
| March 1959 | Valve Guides | It was advised that in the building of replacement engines the works occasionally used 0.01" oversized valve guides. These are identifiable by VG +0.01" stamped on the cylinder head adjacent to one of the stud holes. Over size guides were made available to prospective purchasers, but only in this one size. | - | - | MG/245 |
|  | Gearbox | Close-ratio gearbox introduced as an option. When fitted with this gearbox, engine prefix should be 16GB-Da |  | 1379 | SPL 3rd Ed – F.5 |
|  | Headlights | Asymmetrical headlights introduced on cars for France (cp. 1500 car number 60340) | 1618 | - | SPL 3rd Ed – N.6 |
| April 1959 | Distributor Fitting | Excessive wear on the distributor drive gears indicated a misalignment causing stiffness and excess loading. It was considered necessary to draw attention to the problem which was considered to have stemmed from the incorrect tightening of the clamp. The proper sequence was detailed. | - | - | MG/248 |
|  | Battery Covers | In order to prevent mud, snow, road dust etc., etc, battery covers were made available as optional extras. http://mgaguru.com/mgtech/accessories/at114.htm | - | - | MG/249 & SPL 3rd Ed – N.1 |
|  | Camshaft Installation | As a number of camshafts were being distorted or destroyed during fitting or removal, the proper sequence was further emphasized. | - | - | MG/251 |
|  | Con Rods | Emphasis was placed upon the necessity to ensure that connecting rods are matched by weight. Each rod assembly has a standard weight of llb 15oz 6drms, but may vary above or below by up to one oz. Sets of rods should be identical by weight, and it is vital that any one replacement should not differ from the others by more than 3 drms. If there is no marking on a rod, it is standard. However, a plus sign indicates drams above standard, and a number in isolation indicates drams below standard. The following indicates replacements. To replace. Use only. Std. +l +2 +3 +4 +5 +2 +6 +7 +8 +9 +1O +7 +11 +12 +13 +14 +15 +16 +13 1 2 3 4 5 2 6 7 8 9 10 7 11 12 13 14 15 16 13 | - | - | MG/252 |
|  | Knock Ons | Knock-Ons of bronze with raised face introduced instead of steel Knock-Ons with flat face. | 1826 | - | SPL 3rd Edition – P1 |
|  | Rear Hubs Changed | Rear car hub extensions modified. Identified by either thin or thick casting behind knock on thread. Was AJC5182/3, became ATB7252/3 | 1840 | - | SPL 3rd Ed – H.2 |
|  | Tappet Sleeves | Cast iron sleeves introduced for tappet bores in cylinder head to reduce wear. | - | 1587 | SPL 3rd Ed – A.11 |
| May 1959 | Sun Visor | Coupe, new type of optional extra sun visor. (cp. 1600 Car number 81532) | 2061 | - | SPL 3rd Ed – R.55 |
|  | Tappets | In order to reduce the possibility of tappet failure, the length of the bucket was increased by 0.25” from 1.25" to 1.5" to eliminate risk of fractures. The change was effected from engine No. 1087. | - | 1087 | MG/253 & SPL 3rd Ed – A.11 |
|  | Spark Plugs | In order to reduce the heat condition of the piston crown, the manufacturers recommended a change from the previously incorporated Champion N5 plug, to a Champion N3. | - | - | MG/253 |
| June 1959 | Timing Gasket | At this date the AEH377 timing case gasket was introduced. Being divided into four sections any one of the four could be detached and used separately. This obviated the need to totally dismantle the timing gear each time a new gasket was required. | - | - | MG/257 |
|  | Valve Guides | As a rider to the March 59 notification regarding valve guides, the factory advised that 0.0l" replacements can also be identified by a shallow groove on the outside diameter. | - | - | MG/259 |
|  |  | From chassis No 2192 the front direction indicator light was changed from white to yellow. At the same time, a separate unit was introduced at the rear. Also the Flasher Relay changed. See SPL N.10 | 2192 | - | SPL 3rd Ed – N.7, N.8 |
|  | 1600 Body | Roadster, Introduction of 1600-Type body from chassis No. 2193 (cp. 1600 Car number 68851) with modifications as detailed for MGA 1600. New type of body on coupe from chassis No. 2292. South African CKD twin cam coupes (2189 to 2192 and 2255 to 2258) appear to be exceptions as they were dispatched on 12 June 1959 and incorrectly recorded as roadsters. (Bo Giersa) Also several Wiring Harness changes were made, See SPL N.12 Roadster windscreen has centre catch AFH5457 added see SPL R.11 Roadster Sidescreen Locking handle changed see SPL R.13 Roadster Sidescreens changed from Flap to sliding window type see SPL R.15 Also Boot/Trunk Floor & spare wheel fixings changes on Coupe 2292 See SPL R.6 | 2193 & 2292 | - | SPL 3rd Ed – N.12, R.4, R.11, R.13, R.15 |
|  | Headlights | Cars for the USA fitted Mark VIII sealed beam headlamps in factory, with modified headlamp gasket integral with dust excluder. (cp. 1600 Car number 70222). | 2278 | - | SPL 3rd Ed – N.6 |
|  | Headlight Dipswitch | Dipswitch on left-hand drive cars repositioned for easier operation (this modification introduced on MGA 1600 from start of production) | 2281 | - | SPL 3rd Ed – N.3, R.54 |
| July 1959 | Carburettor Piston | At this date an improved carburettor piston damper was introduced. The original type caused uneven running, and a restriction on maximum speed. The old type can simply be converted to the new, by removing 0.07" from the lower face of the piston. Replacements could, however be purchased. Carburettors with the new type damper have an "0" stamped on the hexagonal damper shaft cap. | - | - | MG/264 |
|  | Hadlights | Asymmetrical headlamps introduced on cars for Sweden. (cp. 1600 car number 72040). | 2418 |  | SPL 3rd Ed – N.5 |
|  | Con Rods | In order to improve balance and to strengthen the big-end a new type connecting rod was introduced. Commencing at engine No. 1343 the new design incorporated a boss at the small end for the purpose of balancing, and a heavier big-end bearing cap. The new type is only interchangeable with the old, in sets. | - | 1343 | MG/262 & SPL 3rd Ed – A.6 |
|  | Flywheel Dowels | To facilitate removal, a 5/32 hole may be drilled from the back of the flywheel into each dowel hole. This will allow the dowel to be knocked out with a 1/8 punch. |  |  | MG/267 |
| August 1959 | Service Parts | Up to this point, the crankshaft, flywheel and clutch could only be supplied as a balanced unit. Due to improved balancing techniques it became possible to supply individual items as replacements, provided that the complete units were balanced after assembly. | - | - | SPL 3rd Ed – A.6 |
|  | Clutch Replacement Parts | In a similar vein to the above, clutch components became available as replacement parts. Provided that the unit is reassembled with the parts in the same relation to each other, the replacement of a thrust or anti-rattle spring will not seriously affect the balance of the assembly. | - | - | MG/275 |
| September 1959 | Anti Roll Bar | As an improvement to the handling, all cars from chassis No. 2275 were fitted with an anti-roll bar. Chassis front extension, front suspension spring pans and wishbones modified to suit installation of anti-roll bar as standard (cp. 1500 Car number 66574) | 2275 | - | MG/278 & SPL 3rd Ed – K.3, K.4, Q.1 |
|  | Rear Axels | An improved design of rear axle splining was introduced. The half shaft and differential splining was changed to the involute type. Items were thus rendered non-interchangeable. | 2371 | - | MG/278 & SPL 3rd Ed – H.1, H.2 |
|  | Tappet Clearance | On this date, the tappet clearance (cold) was reduced from 0.016/0.017" to 0.014/0.015" | - | - | MG/276 |
|  | Speedo Cable | Speedometer cable modified on right-hand drive cars. (cp. 1600 car number 74979). | 2467 | - | SPL 3rd Ed – O.1 |
|  | Air Cleaner | Air cleaners modified to incorporate venturi. http://www.mgaguru.com/mgtech/twincam/tc308.htm | 2468 | - | MG/286 & SPL 3rd Ed – D.5 |
|  |  | A further modification was effected to the oil ring, with the introduction of an expander. All cars were fitted with this new type piston ring, from engine No 2057. Also affected were, 2043, 2053, 2054 and 2055. | - | 2057 | MG/278 & SPL 3rd Ed – A.2, A.3 |
| October 1959 | Statement from MG | It was deemed necessary at this point, due to a number of problems encountered by owners, to issue a statement emphasising the need for care and attention to particular areas. This was however, superseded on November 13th, which follows. | - | - | MG/282 |
| November 1959 | Hood Frame | From chassis No. 2540 the hood linkage was modified in order to enable the hood to be folded up under the tonneau panel, permitting the side-screens to be stowed on top of the cover panel for the batteries. This change in addition permitted full fore and aft seat adjustment. (cp. 1600 car number 78249) | 2540 | - | MG/294 |
|  | Windscreen Washer | Windscreen washer modified. (cp. 1600 car number 80390) | 2544 | - | SPL 3rd Ed – R.12 |
|  | Wiper Wheel Box Assembly | Windscreen wiper wheel box assembly modified. (cp. 1600 car number 80384) | 2545 | - | SPL 3rd Ed – N.11 |
|  | Remedies For Possible Complaints | Extract from 4 Page Service Memorandum 288 produced in which was detailed the remedies for the numerous problems which had so far been encountered. (a) Lack of power - A minimum of 95 octane fuel is desirable; but if this is not available, fuels down to 90 octane may be used. Fuels below 90 must not be used under any circumstances. Correct valve timing is vital, for optimum and trouble free performance. Ignition settings must be maintained at TDC for fuels of 95 octane and above. For 90-95 fuels, the ignition must be set at 2-3 deg ATDC. Ensure that the distributor points are set at 0.015" or this will vary the settings. Carburettor needle OA6 should be used. Tappet settings of 0.014/0.015" must not be exceeded. Sparking plug, type Champion N3 should be used for normal work; N58R for competitions. Carburettor piston dampers must be identical. The new type cannot be used with the old. It is possible with the first 28 cars that the air cleaners move out of their position and foul the carburettor inlet. This may be remedied by spot welding a piece of 0.031" steel strip l 3/4" long, to the air cleaner inner bracket. (b) Piston holing - This is usually the result of over advanced ignition, use of incorrect plugs, incorrect grade of fuel or a combination of all three. The engine is a high compression, high output unit and any additional advance from the required settings will give excessive plug and combustion temperatures, with the consequent possibility of piston damage. In order to ensure correct timing throughout the speed range of the engine it is essential that the distributor advance mechanism be checked using a stroboscopic timing light, and with the vacuum advance disconnected. The amount of advance observed should be within the following: Max advance. Crankshaft rpm 2000 24 deg 3000 26 4000 26 5000 26 If the above figures are exceeded, the static ignition setting may be retarded to a maximum of 3 degs ATDC. If this does not give the required figures, the distributor should be replaced. If the advance is still excessive, the drive gears for the distributor should be renewed. In order to prevent any excessive ignition advance, the vacuum advance pipe should be permanently disconnected, and the union on the manifold blanked off using plug AUC 1289. The vacuum union on the distributor must be left open to the atmosphere. Champion N3 plugs must be used for all normal work. A hotter running plug must not be used as incipient detonation, due to a rise in plug and combustion temperatures might occur and result in very serious damage to the engine. For competition purposes where a high output is sustained, Champion N58R plugs should be used. These will ensure that combustion temperatures are not excessive and allow an increased margin of safety. 100 octane fuel is strongly recommended. (c) Further reference was made to the clamping of the distributor with regard to the excessive wear etc. See April 59. (d) Excessive oil consumption was found to be a problem on the earlier engines. The twin segmented scraper ring fitted with the expander should overcome this problem, with the result that a rate of consumption of 1500 miles per gallon should be realized after about l000 miles. (e) Oiling of spark plugs should not be encountered if the correct type plug is used, and if the expanders are used on the scraper rings. (f) If running on is to be avoided, the combustion chambers must be free of all sharp edges, especially around the plug apertures. Carburettors must be set to give the minimum slow running, with each throttle equally and accurately set, in relation to the other. (g) Tappet failures occur usually in the early type engine fitted with the 1.25" component. Ridging of the bores and the bucket caused the latter to jam. The longer (l.5") bucket overcomes this problem assuming that the bores are in a satisfactory condition. (h) Further attention is brought to the need to install and remove camshafts correctly. See April 59. It is also noted that failures can occur if the recommended tappet clearance of 0.0l4 -0.015" is exceeded. (i) Not shown (j) Vibration in the early power units can only be cured by accurate balancing, (k) Any excessive increase in the end-clearance of the half-speed shaft results in the ignition being advanced. The end clearance must be maintained at 0.003 -0.006" by altering the shims behind the location plate. (l) It is essential to maintain the timing chain tensioner in good order. If the thread is slack or damaged, a new adjuster should be fitted. It was hoped, at the time, that the adherence to the aforementioned would resolve the problems; however, it was not to be, and further major changes were effected at later dates. | - | - | MG/288 |
| December 1959 | Tappets | Tappets modified. They Can be identified by a 45 degree chamfer around the top edge. | - | 2211 | SPL 3rd Ed – A.11 |
| January 1960 | New Distributor | From engine No 2222 and including 2028, 2029, 2038, 2040, 2041, 2188, 2200, 2201, 2202, 2203, 2204, 2205, 2206 and 2209 to 2219 inclusive, a new distributor without vacuum advance was fitted to prevent over-advancing with possible consequent piston failure. It was recommended that when pistons failed, the new type should be fitted. Supposedly retrofitting campaign change to all earlier cars. Coincides with car/chassis No. 2561. Reference was also made to the pointer method of determining drive gear back lash. Old Distributor 1H811, New Distributor AEJ41. | 2561 | 2222 | MG/296 & SPL 3rd Ed – B.1 |
|  | Gearbox Cover | Gearbox front cover bosses modified. | - | 2228 | SPL 3rd Ed – F.1 |
| February 1960 | Service Tool | In order to prevent oil leaks from the gearbox front cover, caused by the seal not being concentric with the primary shaft, a centralizer tool was introduced. It was emphasised that this tool should be used when, for any reason, the front cover plate has to be removed. | - | - | MG/297 |
| 24th February 1960 | Factory Recall | - On this date the factory communicated to ALL its distributors the requirement to change ALL existing cars to the new type distributor without the vacuum advance. This communication both cancelled and superseded all previous references. It was, needless to say, a massive operation involving hundreds of cars and numerous agents throughout the country. Agents and distributors were requested to ascertain the number of vehicles in their stock and in service in their area, and to advise on the quantity of conversion kits required. They were also asked to check their stocks of drive gears in anticipation of a high demand. When the change was effected, it is interesting to note that an allowance of 2 ½ hours was made for changing the distributor, and checking the drive gears. Were it necessary to change the drive gears, an additional allowance of 4 hours was made. Timing was set at TDC for 93 octane; 3 deg ATDC for lower, octanes. | - | - | MG/301 & SPL 3rd Ed – B.2 |
| March 1960 | Piston Oil Ring | After numerous changes of scraper piston ring, from six slot to eight slot, additional expander etc, the top ring was changed. Although a modified type had been introduced at an earlier date, in order to reduce noise, the chrome specification had been retained. On March 23rd, however, a plain ring was introduced so as to improve oil consumption. | - | - | MG/307 |
|  | Crank Bearings | Lead-tin shells were introduced at this time. They differ from the lead-indium, visually, in that the colour, although still greyish, is brighter. Whilst they carry the same part number, they can be further distinguished by a “G” suffix, whereas the lead-indium type is designated PV. The bearings are interchangeable and were offered as approved alternatives. | - | Unknown | MG/309 |
|  | Valve Springs | As valve crash had been experienced on some engines below 7000 rpm, stronger springs were introduced commencing with engine No. 2251. Alternative low compression pistons (8.3:1) introduced, with resulting decrease in power from 108bhp to 100bhp. High and low compression pistons fitted with plain instead of chromium-plated top rings. | 2609 | 2251 | MG/310 & SPL 3rd Ed – A.2, A.11 |
| May 1960 | End of Production | Last MGA Twin Cam made. | 2611 | 2245 | - |
|  |  | Estimated final Twin Cam engine number 2272. | - | 2272 | - |
| l6th August 1960 | Piston Service Part Change | - 8.3:1 pistons were introduced as a further safeguard against piston holing. See 19th January 1961. | - | - | MG/325 & SPL 3rd Ed – A.4 |
| January 1961 | Control Box | An improved wire wound resistance control box was introduced at this date. It superseded the earlier carbon composition type. Although the part numbers remained the same, the new resistance is not interchangeable with the old. | - | - | MG/342 |
|  | Piston Recomendations | Due to persistent piston failure, January 6l saw a full commitment to the 8.3:1 piston. They were made available as standard and oversize. It was recommended that the 9.9:1 type should only be used for competitions purposes, in conjunction with 100 octane fuel. The static setting for the 8.3 is 8 deg BTDC using N3 plugs. Emphasis was once again placed upon the need for accurate setting. The works also advised that from January 6l, all stripped and half engines, all block/piston assemblies, would be fitted with the 8.3’s. | - | - | MG/344 |
| 2nd July 1962 | Windscreen Replacement | As a number of accidents had occurred in the replacement of the windscreen, it was considered necessary to issue detailed steps in tackling the procedure. | - | - | MG/398 |
| 1st March 1963 |  | Finally, finally that is as far as our current information is concerned; if you have your crankshaft thrust washers fitted such that the oil grooves are toward the bearings, they are fitted incorrectly. In March 63 it was discovered that the Workshop Manual is incorrect. This revelation (Possibly a fitting end to this part was made two years after manufacture ceased). | - | - | MG/416 |

SPL = Service Parts List – The MG Series MGA Twin Cam – AKD1296

SPL – First Edition

SPL – Second Edition

# SPL – Third Edition

WM – Workshop Manual (Latest Edition) AKD928B

Further readings and references.

Clausager

Concours Specifications:

http://www.mgcars.org.uk/mgarh/carinfo/concours.htm

Confidential Service Memorandums:

http://www.angelfire.com/rant/mgatwincam/servicememos/index.html

Summary of Service Memorandums:

http://www.angelfire.com/rant/mgatwincam/servicememos/IndexMG.pdf

Production Database:

http://www.mgcars.org.uk/mgarh/twincam.htm

As an interesting note:

Car 501 had engine 101

Car 2611 had engine 2245 Taken from here: Twin Cam Production Database

Car 2608 had engine 2270, so engines numbers are not sequential, or the last car had a bullet proof engine designed to last.

Engine numbers 2260, 2261 and 2262 were sent to South Africa, these 3 engines were then bored out in SA and fitted with factory supplied 1762 cc Hepolite pistons

The last engine produced was 2272 (Unknown what this engine went into Robin ???)

Therefore with all the engine problems experienced by the factory there should be 61 extra engines about somewhere. But somewhere I read there was an order placed for 2500 engines on this basis there maybe 127 extra engines about.

Service Memorandums not referenced:

http://www.angelfire.com/rant/mgatwincam/servicememos/MG331.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG332.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG335.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG338.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG358.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG385.pdf

http://www.angelfire.com/rant/mgatwincam/servicememos/MG394.pdf

Other changes yet to be fully confirmed:

We must stress here, that the factory kept no records of these changes and this is what owners have reported over the years. If their vehicle somehow inherited a part from another vehicle then this may distort the actual cutover period. Also items such as radiators were constructed in batches, so old batch items may have been used up on an add-hoc basis.

If your vehicle appears to be around the cutover point for some of these changes, please report your details to the Yahoo group.

| Date | Topic | Description | Change Effective |  | Supporting Info |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Car # | Engine # |  |
| Unknown | Cross-Member Holes | Front Chassis cross-member holes. chassis 660 does not have holes (see Yahoo Message 16754), chassis 662 does have crossmember holes (see Yahoo Message 16774) | 662 |  | Yahoo Messages 16754 & 16774 |
| Unknown | Door Hinges | Barney identifies the hinge design changed in June 1958 for the Coupes http://mgaguru.com/mgtech/body/bd103b.htm Since 501 was made in April, and by September they were producing chassis 592 which was the first car with removable Inner Wheel Arches. Thus the Door hinges may have changed around or before chassis 600. Chassis 720 Reports Later Hinges | Around 600 |  | Yahoo Messages 16852 & 16859 & 16860 |
| Unknown | Manifold Washers & Nuts | Exhaust Manifold Washers & Nuts changed. |  | 113 & 256 | SPL 3rd Ed – A.21 |
| Unknown | Engine Washer | Washer for Pivot (tab) changed. |  | 329 | SPL 3rd Ed – A.8 |
| Unknown | Pump Strainer Screw | Oil Pump Strainer Screw changed. |  | 499 | SPL 3rd Ed – A.13 |
| Unknown | Brake Master Cylinder Piston | Master cylinder piston changed from ‘O’ ring early type to cup type seal. Chassis 801 has early piston http://www.mgaguru.com/mgtech/twincam/tc208.htm No mention of change to later type in SPL 3rd Edition Page M.2 however! | After 801 |  | Yahoo Messages 9988, & 9966 |
| Unknown | Radiator Part Number | The Part number changed from ARH0098 to ARH0121 Extract taken from Yahoo Files > Components Info > Twin Cam Changes > Radiator Serial Numbers Car #; Radiator #. ???; 811 ARH0098 00138. YD1 593; 059 ARH0121 00169. YD3 720; 808 ARH0098 00032. YD3 801; 801 ARH0098 00041. YDH5 929; 039 ARH0121 00255. YD3 1063; 811 ARH0??? 00130. YD3 1275; 029 ARH0121 00026 and 00027. YD3 1292; 029 ARH0121 00185. YD3 1306; 159 ARH0121 00070. YD3 1613; 089 ARH0121 00064. YD1 1715; ??? ARH0121 00258. YD3 1806; 099 ARH0121 00166. YD3 2176; 059 ARH0121 00124. | After 801 |  | Yahoo Radiator Serial Numbers |
| Unknown | Filler Neck Screws | Filler Neck Screws changed. |  | 596 | SPL 3rd Ed – C.1 |
| Unknown | Engine Vent Pipe | Engine Vent pipe changes & new clip 1G1309 required. |  | 657 | SPL 3rd Ed – A.7, F.4 |
| Unknown | Small End Bush | Part Number change from AEC977 to SEC882 |  | 710 | SPL 3rd Ed – A.6 |
| Unknown | Inlet Manifold | Change from early type Large SU flange for possibly fuel injection to later small flange. |  | 800-900 |  |
| Unknown | Engine Fan Colour | Engine Fan colour is painted either black, red or yellow. Extract taken from Yahoo Files > Components Info > Twin Cam Changes > Fan Colours Car #; Engine #; Fan Colour. YM3/1147; Black. YD3/1171; ?; Black. YD3/1189; 833; Black. YD3/1275; 915; Black. YD3/1306; ??? Robin; Black. YM3/1374; 934; Yellow. YM3/1468; 1057; Black. YD3/1613; 1266; Yellow. YD2/1716; 1352; Red. YD3/1756; 1363; Red. 1402; Yellow. YD3/2170; 1813; Yellow. YD1/2172; 1689; Red. YD3/2298; Yellow. YM1/2417; 2042; Red. YDH5/2010; 2059; Red. YD3/2459; 2069; Yellow. | - | After 915 | Yahoo Fan Colours |
| Late Jan – Early Feb 1959 | Engine Prefix | The engine number prefix changed from 16G to 16GB. It is believed that this coincided with the introduction of the pushrod MGA 1600 which was designated code 16GA, possibly to avoid confusion with warrantee claims. The Highest 16G engine number reported so far is 1055 and the lowest 16G engine number reported is 1020, There may not have been any overlap, probably just errors in reporting in more modern times as most people assume Twin Cam engines were all GB. No changes to the engine other than the stamped aluminium plate on the back of the engine appear to have been made. Chassis #; Prefix1; Engine #; Confirmed. 1341; G; 1007; Yes. 1329; G; 1011. 1358; G; 1019. 1629; GB; 1020. 499; G; 1024; Yes. 1426; G; 1025. 1368; G; 1029. 1399; G; 1032. 1457; G; 1035. 1396; G; 1036. 1472; GB; 1045. 1463; G; 1047. 1423; G; 1050. 1464; G; 1053. 1433; G; 1055. 1468; GB; 1057; Yes. 1471; GB; 1067. 1401; GB; 1072; Yes. | - | 1055 | Yahoo Messages 11678 & 11689 |
| Various | Other Engine Codes | A ”U” or “Da” Stamp was stamped to the engine number plate to indicate Abingdon had converted the gearbox to close ratio. | - | - | Yahoo Message 11681 |
| Unknown | Gearbox Code | Twin Cam gearboxes serial numbers either begin with A or M. The majority of TC gearboxes carry the A code | - | - | - |
| Unknown | Control Box | Control Box change (63Ohms Carbon Type) & (60 Ohms Wire would) |  |  | MG/324 & SPL 3rd Ed – N.4 |
| Unknown | Starter Motor Brushes | Starter Motor Spring Brushes where changed. |  |  | MG/347 & SPL 3rd Ed – N.2 |
| Unknown | Engine Thermostat Housing | Extract taken from Yahoo Files > Components Info > Twin Cam Changes > Thermostat Housing Materials Reported that early cars had aluminum (AL) thermostat housings Cast iron (CI) was introduced on later cars. But where did the change occur? Info below as reported by group members about their cars. Car #; Top; Body. 589; AL; AL. 765; AL; AL. 1275; AL; AL. 1806; AL; CI. 2087; AL; AL. 2355; CI; CI. 2387; CI; CI. 2417; AL; CI. 2574; CI; CI. | After 1275 |  | Yahoo Thermostat Housing Materials |
| November 1959 | Battery Cover | Battery Cover Changes, Now with handle. | 2540 |  | SPL 3rd Ed – R.3 |